BLADED LEVEL GAUGE

Sleeve version

PFG 86 – M

Technical Data

Ambient temperature range:  -10°C to +80°C

Maximum operational pressure:  3 bars

Protection:  IP65

Weight:  2,4 kg

Power supply available:

24 – 48 Volts AC / 110 – 220 Volts AC / 24 Volts CC

Electric motor absorption:  4 Watts

Electric connection:

Pin 1: Common power supply

Pin 2: Power supply 24VAC or DC or 110VAC (according to the model)

Pin 3: Power supply 48VAC or 220VAC (according to the model)

Pin 4: Not used

Pin 5: NC Contact (10A-250V)

Pin 6: Common contacts

Pin 7: NO Contact (10A-250V)

Motor connection:

| 24/48 V AC | Motor cover yellow | Motor cover Orange |
| --- | --- | --- |
| Pin 1 switch PIENO | BROWN | BLUE |
| Pin 2 of wiring plate | BLUE | GREEN |
| Pin 3 of wiring plate | RED | WHITE |

| 110/220 V AC | Motor cover yellow | Motor cover Orange |
| --- | --- | --- |
| Pin 1 switch PIENO | YELLOW | BLUE |
| Pin 2 of wiring plate | RED | RED |
| Pin of wiring plate | BLACK | WHITE |

| 24 V CC | Motor cover yellow | Motor cover Orange |
| --- | --- | --- |
| Pin 1 switch PIENO | BROWN | BLUE |
| Pin 4 of wiring plate | BLEU | GREEN |
| No Connect | RED | WHITE |

BLADED LEVEL GAUGE

Sleeve version

PFG 86 – M

Parts list - Assembly

#### Cover

#### Identificaton plate				ref : PS PFG86

#### Switch						ref : FIN PFG86

#### Wiring plate (contacts + 2 switchs)	ref : STAF PFG86

#### Motor-réducer (24/48V ou 110/220V)	ref : MOT PFG86

#### Spring						ref : MOLLE PFG86

#### Complet main transmission

#### Main bearing proof				ref : CUSC PFG86

#### Cable gland ½ gas thread			ref : PE PFG86

#### Body

#### Main bearing proof				ref : CUSC PFG86

#### Circlips					ref : CIR PFG86

#### Ring proof					ref : CORT PFG86

#### Blade PVC					ref : AC4 PVC1